Clerk sign on/off system

ACTIVATION

The system can only be activated using CONSTRUCTOR version "1.7.6 Build 5" or later. In the "Setup->Clerks" menu you can activate the "Clerk Time" option. When this option is set additional space will be reserved for each clerk to store 10 sign on/off times and 4 periods where the total time is stored. Furthermore you need a SIGN ON/OFF key on the keyboard. This is a new FIXED key with KEYCODE 2098.

Note that when the system is activated you MUST use sign on/off and a SIGN ON/OFF key must be present on the keyboard.

OPERATION

When a clerk is not signed on he cannot make transactions on the register but he can still take reports.

The procedure to sign on/off is:

1 - Select clerk by key or lock.

2 - Enter the CLERK SEQUENCE number on the key SIGN ON/OFF. The clerk sequence number is the clerk record# in the clerk file. A normal clerk is only allowed to enter his OWN sequence number. A clerk which is also MASTER clerk can enter any clerk sequence number so he can also sign on/off other clerks.

3 - The register will now show on the screen the Clerk name and the current status: SIGNED ON or SIGNED OFF (fixed message).

4 - Now press the SIGN ON/OFF key again without entry. The register will now SIGN ON when the clerk was SIGNED OFF. When the clerk was SIGNED ON the register will SIGN OFF the clerk. The register will store the current time and will issue a ticket on the receipt printer and also print on the journal if available. When the clerk is SIGNED OFF the register will also UPDATE the total time in the clerk time report.

5 - Now the clerk is allowed to make transactions.

The system assumes that when a clerk is SIGNED ON he will be SIGNED OFF within 24 hours. When a clerk stays signed on for more then 24 hours the register will not count the days!!

REPORTS

When the system is active there are ALWAYS 4 periods activated which can be used as DAILY, WEEKLY, MONTHLY and YEARLY totals. There is no need to take the other period reports when the additional periods are not required.

The CLERK TIME report can only be included in the LINKED REPORT LIST of a USER REPORT of the type CLERK.

When programming using the register keyboard, the report type is 15.

The register will report the total time of the period as set in the USER REPORT. When the report is of PERIOD DAILY the register will also print the last 10 sign on/off times. 	The report can also be taken as SYSTEM REPORT 114 where 14 is the type and 1 means clerk.

SPECIAL OPTIONS

- When the CLERK SIGN ON/OFF sytem is active, the register will also issue a ticket and print on the journal a message "NEW TIME/DATE" when the time or date is changed.

- When a CLERK name starts with '*' (example "*JAN") the clerk can only SIGN ON/OFF and take reports but cannot make transactions when he tries error 46 (NOT AUTHORIZED) will be displayed.

- When in CONSTRUCTOR "General->Clerk Options-> Clerk Sign On/Off in network" (SYSTEMFLAG 123 BIT 0 (value 1)) is set the register will also sent

the SIGN ON/OFF command to the other registers in the network. It is possible to SIGN ON/OFF on any register in the network but you must be sure the clock is set to the same time because the time of the used register is taken. It is advised that the SIGN ON/OFF is done on the same register. The report can be taken on any register

and in Z-position it will be automatically cleared in the other registers.

- When in CONSTRUCTOR "General->Clerk Options-> Sign On/Off before Clerk

Z-report" (SYSTEMFLAG 123 BIT 1 (value 2)) is set, the register will check if the clerk is still signed on when a Z-report is requested 	and will give an error.

- When in CONSTRUCTOR "General->Clerk Options-> Sign On/Off with Clerk

Z-report" (SYSTEMFLAG 123 BIT 2 (value 4)) is set, the register will automatically sign off the clerk when it was still signed on.

- When the non of the above report options is set the clerk will stay signed on after a Z-report.  /* BIT 2, if set then automatic sign off with clerk Z-report */

MESSAGES

The following messages are used for the CLERK SIGN ON/OFF System:

"Signed On       " -> used on screen and printer

"Signed Off      " -> used on screen and printer

"First Sign On ! " -> used as error message

"First Sign Off !" -> used as error message

"Total Time     :" -> used in report

"New Time/Date   " -> used on screen and printer when time/date changed.

These messages can be programmed in CONSTRUCTOR "General->Program texts->System Messages->108 to 113" respectively.

The message in CONSTRUCTOR "General->Program texts->Report Names->16"

is used as text in the CLERK TIME report.

COMPUTER REPORT

The clerk time report can also be taken by computer as part of a USER report or as a SYSTEM report. There are 2 new record types added for this purpose.

Record 117 of the type TEXT will contain in the DATA field the SIGN ON and SIGN OFF time at fixed locations. This record will only be sent when a DAILY report is taken.

"117,  1,  0,  1,    0,       13:56        5:07"

Record 118 of the type TEXT will contain the TOTAL TIME for the requested period.

"118,  1,  0,  1,    0,                   15:11"

There is no new driver required for this new record types.