ELLIX20 Logo Setup

You will need

- Run ImageDownloaderV1.00.exe

- Select your Ellix20 printer from the drop down list on the SETUP PORT tab.

- Hold down the printer feed button then hold down the power button at the same time. Do not release the buttons until the “Settings” have completed printing. The printer will turn off automatically when complete.

- Confirm that the printed settings match the relevant settings on the SETUP PORT tab.

- Go to the SELECT BITMAP tab. Use the OPEN FILE button to browse for the bitmaps that you wish to transfer to the printer.

Note – Bitmap files must be (384 x 196 pixels) or (512 x 196 pixels), and they must be 2 colour images. Check the ‘Sample Images’ folder included with your software for examples.

- Go to the PREVIEW BITMAP tab to confirm your selections.

- Confirm that the printer is powered ON and connected directly via the serial cable to your PC.

- Go to the DOWNLOAD BITMAP tab to begin the download.

- When the progress meter completes, select OK.

- Go to the PRINT BITMAP tab to print your logos. If the progress bar continues to run after the printing is complete, you may select CANCEL.

- If the logos fail to print, please follow the procedure again from Step 1 above.