How to use the Datalogic JavaPOS Driver.

JavaPOS is a standard interface for POS devices.

The new driver JavaPOS made by Datalogic is the version 1.0.0.20051229. Improvement of this release are:

- Support for DLL2020 (the same driver is now able to support both the old DLL2020 and the Datalogic POS compatible devices)

- Able to send a generic command (using the DirectIO function) to the device

- Compatible with the JavaPOS rules 1.9

Now, a quick explanation in order to use correctly the JavaPOS driver.

It is necessary to install the following SW (you can find instruction in the readme file included in the JavaPOS package)

- Java2 runtime environment

- JavaPOS Config Loader

- Java communications library

- Xerces XML Parser

You can find instruction about how to install the SW in the “readme.txt” file included in the JavaPOS packet. See the instructions displayed to have a right installation.

# Datalogic JavaPOS Configuration utility

After the installation, the device must be registered in the driver. From a prompt window, you have to type the command “java com.datalogic.jpossetup.ConfigApp”. Now, a window appear in your host:

You have to click, on the device detection side, depending by the device you need to use: if you have to use the DLL2020, you have to click in the relative icon (DLL2020); the icon “other” refer to the Datalogic standard POS devices (you can select the auto detection or you can select manually the COM port used). The device, initially, must be in default RS232 interface.

After this, a splash screen appear, and the device check start; if the device is correctly detected, a sequence of beep can be hear, and the blue inscription appear. Now, you can register the device in the driver and you can work.

In order to check if the system is working, or if there are some problem in the customer environment, you can check immediately the system, using the “Test” button. Clicking it, a new window is show:

This is the suite for test if the group (barcode reader, driver, customer host) is working fine. You have to click on the follow:

- Logical name (select the right logical name of the device, if you have more than one device connected)

- Open

- Claim (at this point you have to hear a sequence of beep from the reader: this is the JavaPOS configuration printed in the reader)

- Device enabled (now the red laser light is emitted when you press the trigger)

- Data Event enabled (Now you can see data when the device read barcodes)

In order to close the communication with the device, the commands sequence is: Data event disabled, Device disabled, Release, Close. Now the device is free.

Using the DirectIO command, you can send to the reader all the possible command (without driver control! This mean that you have to be sure about the command you need to send).

# Device substitution

It is important that, if you need to change device in the customer site with a new one (ex: change a broken Gryphon D100 with a new Gryphon D100), you can simply remove the old reader, program the new one with the JavaPOS configuration, and connect in the customer system. It is not necessary to register again the new device.

The change is transparent for the customer system and it work (if you need to change a device with a new, but different one, the register procedure is necessary, if the new device was not connect before in the customer system).

The command for the JavaPOS configuration (of the STD Datalogic readers) is “$+CL99EC0CF3EA0102$-” and “$+EA130D0A03EB1CR16564$-” (split in two barcodes in order to read them easily)

This command set:

- Serial communication timeout and ACK/NACK protocol;

- Header and Terminators setting;

- Serial trigger lock;

For the old DLL2020, the commands are:

-   ENQ (Ctrl-E) + Shift In (Ctrl-O) +  ‘!’  // Enter in LOCAL mode

- Progmode 	// Enter in Configuration mode

- 14420 	// Set Header

- 13102 	// STX

- 13100	// Header OK

- 14421	// Set Terminators

- 13135	// CR

- 13103	// STX

- 13100	// Terminators OK

- exit

- save –m=stack  // Save cfg

- ENQ (Ctrl-E)  // Exit from the LOCAL mode